Thermal treatments

List all the thermal treatments needed, the type of heat exchangers and the mode of use (i.e. co-current vs counter current).

HEAT EXCHANGERS

Steps; Heat exchangers; Type; Mode. 1. 2. 3. 4. 5.

Determine the time of pasteurization to decrease of 5D the initial amount of microorganisms. Assume as reference pathogen the vegetative cells of Escherichia coli (D(70°C) = 2 s; Z = 5°C). In case of sterilization processes, the target is the reduction of 12D of the spore of Clostridium botulinum (D(121) = 0.2 min; Z = 10°C).

Report here below all the calculation and the final result:

MICROBIAL REDUCTION

Determine the loss of thiamine (D(120°C) = 9000 s, Z = 25°C):

THIAMINE LOSS

Assuming that the initial pectinase activity is 10 mmol of pectin per minute per kg of produt, determine the final activity after the thermal processes. (D(80°C) = 10 min,  e Z = 10°C):

ENZYME DENATURATION